# MicroBlaze-V-Zephyr-Kria-OpenAMP

Getting Started Guide, document revision v1.0

## Preparing the KR260 and Sensor Setup

- Purchase the KR260 and Pmod Accessories

  - KR260 Board , power adapter , 16+ GB of SD Card – Purchase Link from AMD

  - Pmod Acl – Digilent Link

  - Pmod TMP2 – Digilent Link

  - Pmod GPIO 8 bit LED – Digilent Link

  - Pmod UART – Digilent Link

  - Few Male to female jumpers – Amazon or can be purchased from any electronics shop.

- Prepare the KR260 with power adapter , 16GB+ of SD card and burn the Kria-Ubuntu 22.04 LTS OS image on the SD card.

  - Download image

  - Burn using Balena etcher

- Connect the sensors

  - Pmod Acl - Accelerometer

    - Connect to Pmod1.

  - Pmod TMP2 – Temperature sensor

    - Connect to Pmod3.

  - Pmod GPIO – 8 bit LED

    - Connect to Pmod4.

  - Pmod UART and PL Reset

    - Connect to Pmod2.

    - You can follow following VIVADO Constraint section  for connection.

# UARTlite

# UART_txd PMOD #2 pin 2 (connect rx of the uart device to pin 2)

set_property PACKAGE_PIN H11 [get_ports UART_txd]

set_property IOSTANDARD LVCMOS33 [get_ports UART_txd]

# UART_rxd PMOD #2 pin 4 (connect tx of the uart device to pin 4)

set_property PACKAGE_PIN G10 [get_ports UART_rxd]

set_property IOSTANDARD LVCMOS33 [get_ports UART_rxd]

# Reset button

# PMOD #2 pin 1

set_property PACKAGE_PIN J11 [get_ports ext_reset]

set_property IOSTANDARD LVCMOS33 [get_ports ext_reset]

- Connect the Pmod-UART and KR260 UART on the board with PC. You can use Mobazterm/Teraterm in Windows or Gtkterm in Linux.

  - Connect the baudrate of 115200 for both of the UARTs

- Connection now looks like:

- Now boot the KR260 board.

## Download and Update the Kernel

- Download the updated kernel from here using wget command at KR260 (or do the download at PC and SSH copy or USB disk based copy can be done).

- Now follow this command to update the kernel at Kria-Ubuntu

- Reboot the board

## Download the Firmware and Host Code

- Download the firmware from here – Github link

  - Can also do the wget for the firmware file only or perform SSH copy or USB drive based copy

- Copy the firmware at the /lib/firmware/Xilinx

- Now load the firmware using xmutil command

## Application run

- You can connect the KR260 via SSH (for multi terminal access) as like here:

- Now, run the OpenAMP host app on the Kria-Ubuntu terminal.

- Now unload the kria app and re-load it.

- Now you must able to see the output as like in above picture.

## Revision History

| S.N | Revision Number | Revision Details | Author |
| --- | --- | --- | --- |
| 1. | v1.0 | Initial Revision | Krishna Gaihre and Sanam Shakya |